Introduction to Heat Pump Scheduling and Graph Message Refinement

Physical model candidate sampling and decision training

This document explains how to schedule heat pumps and thermal storage to meet heat demand, follow an electrical power reference and minimize the complete electricity bill. It introduces the physical model first, then shows how to generate candidate actions and evaluate them with a graph neural network (GNN). Storage location, connection choices and equipment operating history determine how each current action affects later operation.

The computation method starts with a complete GNN as a reference evaluator. A cheaper branch scores every candidate, and selected messages are replaced by complete messages within a time budget. Candidate density controls how many actions are explored; message fidelity controls how much computation each evaluation receives. The reference plant calculates physical consequences, and a complete trajectory check determines which action is ready for execution.

Parts I to III introduce the system, its available information, service requirements and electricity cost. Part IV explains candidate sampling, energy messages, decision training and the allocation of computation. The central idea is to spend detailed neural computation where it is most useful for preserving the complete evaluator's action choice. The method is assessed through decision loss, execution time and the electricity bills of accepted plans.

Common notation

Table 1  Main quantities and units

| Symbol | Meaning | Unit or type |
| --- | --- | --- |
| i,j,t,T | Pump index, tank index, interval index and terminal state boundary | Indices; t=0,…,T-1 |
| Δt | Duration of one control interval | h |
| Pi,t,Qi,t | Pump electrical input and produced heat | kW electric; kW thermal |
| vij,t,qij,t | Charging assignment and heat on a pump to tank edge | Binary; kW thermal |
| Sj,t,Cj | Tank energy and effective heat capacity | kWh thermal; kWh/K |
| dj,t,Dt | Useful heat delivered by a tank and aggregate requested heat | kW thermal |
| ct,Ptref | Given tariff and aggregate power reference | Currency/kWh electric; kW electric |
| εtP,εtQ | Fixed service tolerances | kW electric; kW thermal |
| xt,at | Physical state and complete physical control action | Vectors defined in Part I |
| Vt*(x),Jt*(x,a) | Optimal remaining bill and cost after fixing the current action | Currency |
| ye,kL/H | Learned part of an edge message with exact energy coordinates retained | Latent coordinates |
| b,Nb,fjb,t | Building index and count; permitted delivery flow | Index; count; kW thermal |
| JH(k),J𝒮(k) | Complete and mixed scores of joint candidate k | Currency |
| π,𝒮,Cπ | Policy (physical control in Part III; computation in Part IV), refined positions and cumulative time | Policy; set; time |
| Jcont(k) | Verified simulated continuation-cost label for candidate k | Currency |
| δSnum,δPnum | Fixed terminal acceptance bands | kWh thermal; kW electric |

Indices i, j and b run over pumps, tanks and buildings. The interval index is t=0,…,T-1, and T is the terminal state boundary. Constraints apply to every relevant device and interval. Bold or grouped quantities collect device values. A superscript in parentheses, such as (k), identifies a candidate; + marks its successor state. Descriptive subscripts and superscripts label a quantity's role. Hats mark predictions or estimates, and overbars mark capacity limits except where interval bounds are stated explicitly.

An empty sum is zero. The operators min and max select the smallest and largest listed values. The symbols ∩, × and ∖ denote set intersection, Cartesian product and set difference; ⊆ denotes set inclusion allowing equality. Vertical bars denote absolute value for a scalar and the number of elements for a set. Each section defines additional symbols next to the relevant equations.

Part I Physical Model and Service Constraints

1 Heat paths and control variables

There are Nh heat pumps, Ns single-temperature tanks and Nb buildings. A directed charging edge (i,j)∈Ech permits pump i to charge tank j. A delivery edge (j,b)∈Edel permits tank j to supply building b. The sets Ech and Edel contain the allowed charging and delivery connections. Heat follows pump-to-tank and tank-to-building paths. Separate branches support charging and delivery in the same interval. Building demand is represented by its aggregate total.

An action contains binary on/off decisions ui,t, electrical powers Pi,t, binary receiver assignments vij,t and useful delivery flows fjb,t. Tank delivery dj,t is the sum of its outgoing delivery flows. An operating pump uses exactly one permitted charging edge:

∑j:(i,j)∈Echvij,t=ui,t,  ui,t,vij,t∈{0,1}.	(1)

An on/off variable equals one when the pump operates and zero when it is off. An assignment equals one when that connection is selected and zero otherwise. Unavailable assignments are fixed at zero. Several pumps can charge the same tank. An off pump has zero electrical power and heat output; its previous power and remaining minimum off time remain part of the decision state.

Delivery flows are nonnegative on allowed edges and zero elsewhere. Tank delivery and building receipt are obtained by summing these flows:

dj,t=∑b:(j,b)∈Edelfjb,t,  yb,t=∑j:(j,b)∈Edelfjb,t,  fjb,t≥0.	(1a)

Here yb,t is the useful heat power received by building b. The service requirement uses total building demand. For fixed tank delivery totals, equipment states and external inputs, reallocating heat among allowed building connections preserves physical cost and feasibility. The complete, coarse and mixed evaluators therefore use tank delivery totals and aggregate demand as scoring inputs. Individual building flows provide the delivery record. A tank with no allowed building connection has zero useful delivery capacity.

For an active pump, the receiving temperature is that of its selected tank. Its actual heat production is

Qi,t=κi(Pi,t,Pi,t-1,Ti,trec,Ttsrc,Tisup)Pi,t,  qij,t=vij,tQi,t.	(2)

The coefficient of performance (COP) κi is the ratio of produced heat power to electrical power. Trec, Tsrc and Tsup denote receiving-tank, heat-source and pump-supply temperatures. Supply temperature is prescribed. The plant relation determines Qi,t, and qij,t is the heat power sent along the selected charging edge. An off pump has zero output and bypasses the active-pump COP calculation. Application of this model requires a specified numerical COP law, operating domain and treatment of previous power.

2 Storage and delivered heat

Tank energy uses a fixed temperature reference:

Sj,t=Cj(Tj,ttank-Tref).	(3)

Sj,t is tank j's stored thermal energy. The effective heat capacity Cj converts temperature difference into energy, and Tj,ttank is the tank temperature. The lower inventory limit corresponds to the minimum permitted tank temperature; energy above this limit is available for use. The reference Tref sets the energy origin. Required delivery temperature is a separate physical setting.

The interval heat balance is

Sj,t+1=Sj,t+Δt[ηjc∑iqij,t-dj,t/ηjd]  -Δt UAj(Tj,ttank-Ttamb).	(4)

Here ηjc and ηjd are dimensionless charging and delivery efficiencies in the range (0,1], and UAj is the tank loss coefficient in kW/K. Edge heat qij,t is measured before charging loss; dj,t is useful heat delivered to users. Each efficiency applies once on its own branch. Multiplication by Δt converts power into energy. The ambient temperature is Ttamb. The resulting Sj,t+1 is the successor inventory used in the capacity check.

The relevant capacities are

Sjmin≤Sj,t≤Sjmax0≤qij,t≤qijvij,t∑iqij,t≤Qjch0≤dj,t≤dj.	(5)

Here Sjmin and Sjmax are tank inventory limits, qij is the charging-edge heat-power limit, Qjch is the tank's total charging-power limit, and dj is its useful delivery-power limit. These limits apply to each device and connection, including in a fully connected graph. They determine how rapidly heat can be stored, delivered and replenished at each location.

3 Equipment memory and shared heat source

Operating powers and ramps satisfy

Piminui,t≤Pi,t≤Pimaxui,t-Ridown≤Pi,t-Pi,t-1≤Riup.	(6)

Here Pimin and Pimax are the operating-power limits. The quantities Ridown and Riup bound power decreases and increases over one interval, in kW. They include starts from zero and stops at zero. Minimum on and off durations are fixed device constraints, represented by remaining interval counts. An active lock retains the previous mode. After a switch, the remaining lock equals the corresponding minimum duration minus the interval just executed; it then decreases towards zero. Electricity cost in (11) supplies the operating objective.

The heat extracted from the shared source is heat output minus electrical input:

0≤∑i(Qi,t-Pi,t)≤Qtsrc.	(7)

Here Qtsrc is the available shared-source heat-extraction capacity in interval t, measured in kW thermal. Equation (7) balances heat production and electrical input for the specified pump model, whose internal heat storage and residual output after shutdown are zero. The aggregate extraction limit applies jointly to all pumps, alongside the receiving limits of each tank.

The physical state is xt=(St,Pt-1,ut-1,ltlock), and the joint action is at=(ut,Pt,vt,ft). Here St collects tank inventories, Pt-1 collects previous pump powers, ut-1 collects previous on/off modes, and ltlock collects remaining minimum-duration obligations as interval counts. The vectors ut, Pt and vt collect current modes, powers and assignments. ft collects allowed delivery flows, and 𝒯t is the reference-plant transition: xt+1=𝒯t(xt,at). At the first interval, previous powers and modes are supplied by the initial conditions. Together, inventory, previous power and equipment locks determine the legal current actions. A complete continuation also satisfies the requirements in every remaining interval.

Part II Available Information and Prediction Models

1 Reference plant and heat production prediction

The main scenario uses a specified deterministic reference plant. Tariffs, requested heat, power references, source and ambient temperatures, and source capacities are known at episode start. The plant version, initial state, device limits and external trajectories define each case. This setting supports direct comparison of candidate evaluation methods under the same physical conditions. An extension with forecast error requires its own treatment of uncertain future inputs.

A heat-production predictor supports proposal generation. For an active pump, a history encoder summarizes observations into the latent vector hi,t, and a decoder evaluates the proposed power and receiving tank:

κi,t=Fϕi(hi,t,Pi,t,Ti,trec,Ttsrc,Tisup),  Qi,t=Pi,tκi,t.	(8)

Here Fϕi is pump i's prediction decoder with parameters ϕi. The hatted outputs are the predicted COP and heat power. Proposed power and receiving temperature are decoder inputs. For multiple future steps, each candidate propagates its own predicted states. The reference plant supplies the current physical calculations and complete-plan checks used in Part IV.

The initial implementation generates proposals with the known plant. Equation (8) provides an optional learned predictor for heat production. The GNN in Part IV has a different target: the remaining electricity bill. Known future demand enters as an input sequence. History encoders use observations available before the decision; a demand forecast model belongs to the extension with uncertain inputs.

Part IV uses three further predictions: continuation cost in currency, latent-message error in representation space, and incremental execution time. The runtime predictor takes workload and cache information as inputs. These models guide computation and candidate ordering; reference-plant checks determine physical acceptance.

2 Prediction training and data separation

Prediction training uses measured or simulated input-target pairs, with input history preceding the target observation. A fixed power scale makes the heat-production loss dimensionless:

ℒpred(ϕ)=1|𝒟|∑(i,t)∈𝒟(Qi,t-Qi,tobsQiscale)2.	(9)

D is the training set of pump-interval pairs, and Qiscale>0 is a fixed heat-power scale for pump i. The denominator |𝒟| is the number of training pairs, Qi,tobs is the target heat power, and ϕ collects the prediction-model parameters. The loss ℒpred averages squared normalized prediction errors. Equation (11) separately defines the electricity-cost objective, and equation (10) defines service acceptance. Off-mode heat is fixed at zero.

Training, validation and final evaluation use separate trajectories and graph configurations. Optional online updates use observations obtained after executed actions, with simulated targets identified as simulated. Model parameters, preprocessing, history and update memory remain fixed during each planning call. The initial message-refinement comparison also fixes the complete evaluator, coarse branch and error predictor throughout testing, so measured differences describe the computational approximation.

Part III Electricity Cost and Fixed Service Requirements

1 Electrical and thermal service throughout the episode

The aggregate power reference applies in every interval of the episode. Service is defined by

|∑iPi,t-Ptref|≤εtP|∑jdj,t-Dt|≤εtQt=0,…,T-1.	(10)

Here Ptref is the given aggregate electrical-power reference, Dt is requested useful heat power, and εtP and εtQ are fixed nonnegative service tolerances in their respective power units. The aggregate heat term in (10) also equals ∑j,bfjb,t. Flows are allocated over allowed delivery edges within tank capacities. Service is accepted using the aggregate power and heat bands throughout the episode.

Power reference, tariff and temperature settings are external inputs. Pump powers and tank temperatures evolve under control. The controller uses tariff variation to choose the timing of electricity use within the feasible service bands.

2 Complete bill and common terminal condition

The interval and episode costs are

gt(at)=ctΔt∑iPi,t,  Jπ=Eπ[∑t=0T-1gt(at)].	(11)

gt is the interval electricity cost and Jπ is the expected complete bill, both in currency. The tariff ct is measured in currency per kWh electric. In Part III, π is the physical control policy that chooses actions from available information, and 𝔼π averages over any policy randomness. The reference plant and external inputs are deterministic. The bill includes all preparation and recovery within the episode, with equal weight on costs at every interval.

Every controller starts from the same physical state and is required to finish in the common terminal set XT. The reference terminal requirements are

|Sj,T-Sjtar|≤δSnum|Pi,T-1-Pitar|≤δPnumui,T-1=uitarli,Tlock=0.	(12)

Here Sjtar, Pitar and uitar are the prescribed final tank inventory, pump power and on/off mode. The lock value li,Tlock is the remaining minimum-duration obligation at the terminal boundary. The fixed bands δSnum in kWh and δPnum in kW set numerical acceptance tolerances. Continuation labels, candidate checks and backup plans share these targets and obligations. Zero bands give exact terminal equality. With positive bands, two accepted plans can differ by up to 2δSnum in a tank inventory and 2δPnum in a final pump power. Per-device terminal deviations accompany the bill comparison so that this remaining variation is visible.

Terminal inventory is checked separately for every tank. The endpoint requirements specify the commitments included up to T. In Part IV, both message fidelities retain the same normalized physical coordinates and differ in the learned vector ye,k. Latent error, score error and runtime error are measured in representation space, currency and time, respectively. Service and terminal checks use the physical units and tolerances defined in (10) and (12).

When εtP=0 in every interval, the power reference fixes the complete electricity bill in (11), and the scheduling task becomes a feasibility problem. Positive power tolerances create a range of permitted aggregate powers. The economic objective is to find the least-cost trajectory satisfying the equipment, heat-service and terminal requirements.

3 Optimal remaining cost and continuation labels

Let Ft(x) be the set of remaining action trajectories starting from physical state x, satisfying the plant and remaining service requirements, and reaching XT. The optimal remaining cost and the cost after fixing the current action are

Vt*(x)=inf(at,…,aT-1)∈ℱt(x)∑s=tT-1gs(as),Jt*(x,a)=gt(a)+Vt+1*(𝒯t(x,a)).	(13)

The value of an empty continuation set is +∞. At a valid terminal state, remaining cost is zero. Vt* is the infimum of feasible remaining bills, and Jt* is the corresponding value after fixing the current action. The operator inf gives the greatest lower bound over feasible trajectories; s is the summation time index, and as is the action at that time. The star marks an optimal value. The symbol Qi,t retains its physical meaning as pump heat power.

The action-value decomposition in (13) applies to actions satisfying current physical and service requirements. A current violation gives Jt*(x,a)=+∞. Feasible current actions proceed through the transition and remaining-cost calculation.

The quantities Vt* and Jt* define the physical optimum and provide quality references when reliably computed. The practical learning workflow uses the verified continuation bills in (14a) as labels, trains a complete GNN to predict them, then approximates that frozen evaluator under a computation budget. Its score follows the current-cost-plus-future-cost structure of (13). Complete-plan acceptance determines execution, and its search and verification time enter the total control time.

Part IV Budgeted Message Refinement for Joint Action Evaluation

1 From physical requirements to graph messages

The message-refinement method approximates the frozen complete GNN within a computation budget. Both branches use the same exact current-step energy balances. The labels L, H and 𝒮 denote coarse, complete and mixed neural quantities. Refinement replaces selected learned message vectors and updates the resulting candidate scores.

Three physical requirements shape the representation. Resource contributions are attributed to edges and added in energy balances. Shared tanks and the shared source enter through the context of the complete joint action. Aggregate building demand is represented through tank delivery totals, preserving equivalent delivery allocations. These choices identify the exact message coordinates, the learned interactions and the computations available for refinement.

The design connects problem requirements to model operations, following the approach illustrated by Proactive Resource Request for Disaster Response. GNN-FiLM and Heterogeneous Graph Transformer provide examples of conditioning and relation-dependent parameters; FluxGNN illustrates structural encoding of physical requirements. The present method combines energy attribution, delivery equivalence and selective message computation. Allowed charging connections can number NhNs, while equation (1) limits active charging edges to one per running pump. Scoring covers the allowed connections, including background information on zero-flow edges. Shared information is cached, and comparisons with inexpensive neighbourhood aggregation assess the value of this additional neural computation.

2 Joint candidates and the frozen reference

At interval t, a common generator produces complete current actions indexed by k=0,…,K. The pool has K+1 candidates. Candidate zero is the first action of the currently verified backup trajectory. Every candidate specifies pump modes, powers, receivers and allowed delivery flows, starting from the same actual state xt:

𝒜t={At(0),…,At(K)},  xk+=𝒯t(xt,At(k)).	(14)

Here 𝒜t is the current candidate pool, At(k) is its candidate action, and xk+ is candidate k's hypothetical successor. Candidate generation, current physical checks and exact interval costs are shared by the complete, coarse and mixed evaluators. Current legality admits an action to the pool; complete-trajectory acceptance later determines its readiness for execution.

Offline labels are generated by a fixed continuation procedure. It holds the candidate's current action fixed and searches for the remaining actions under declared rules, a search budget and a randomness policy. Equivalent building allocations use the same representation and continuation treatment. A trajectory that passes all equipment, service and terminal checks supplies the label

Jcont(k)=gt(At(k))+∑s=t+1T-1gs(ascont,(k)).	(14a)

Here s indexes future intervals, ascont,(k) is a verified action in the continuation, and the label cont identifies this procedure. The quantity Jcont(k) is the simulated remaining bill, including the exact current cost. A verified feasible bill is at least the physical optimum in (13). At the last interval, the future sum is zero. Unresolved searches are recorded as UNKNOWN, and their frequency is reported with label coverage. Supervised cost fitting uses the verified labels.

The complete evaluator H is trained on these labels and then frozen. It consists of context construction, a message block, node updates and a cost readout. It predicts continuation cost to order candidates before the more expensive trajectory search. Once a complete trajectory passes verification, its bill is calculated directly from its actions.

Nested sampling of candidate actions

Candidate generation first selects the discrete equipment mode, then samples continuous powers and delivery flows for the whole system. At the fixed state xt, the construction is

σ=(u,v)∈Σt,  ζ∈[0,1]dσ,  at=Mxt,σ(ζ).	(14b)

Here σ contains all binary on/off and receiver decisions. The set Σt is the declared catalogue of modes satisfying allowed connections and equipment locks. The vector ζ contains normalized sampling coordinates, and dσ is their fixed nonnegative integer count for that mode. The map Mxt,σ converts a hypercube point into a complete action. Conditional allocation and physical checks determine which mapped actions enter the pool. Small systems can enumerate every discrete mode; for a restricted catalogue, evaluation records the modes covered. Discrete modes are selected directly from the catalogue.

First choose powers consistent with the mode and equation (6). Suppressing t on current controls and bounds gives

Pi=max{Piminui, Pi,t-1-Ridown},Pi=min{Pimaxui, Pi,t-1+Riup}.	(14c)

The lower and upper bounds Pi and Pi combine operating limits with the ramp from Pi,t-1. For an off pump, a feasible interval consists of zero. An empty interval rejects the mode. To satisfy the electrical service band in (10), first select total power and then allocate it among pumps:

[LP,UP]=[Ptref-εtP, Ptref+εtP]∩[∑iPi, ∑iPi],pΣ=LP+(UP-LP)ζΣP.	(14d)

Here LP,UP are the lower and upper bounds on total power, pΣ is the selected total, and ζΣP∈[0,1] is its sampling coordinate. An empty intersection rejects the mode. A single-point interval fixes the total. Allocation then proceeds in a fixed pump order:

ρiP=pΣ-∑i′<iPi′,Picond=max{Pi, ρiP-∑i′>iPi′},Picond=min{Pi, ρiP-∑i′>iPi′},Pi=Picond+(Picond-Picond)ζiP.	(14e)

The quantity ρiP is the power remaining before pump i; i' indexes earlier or later pumps in the fixed order. The superscript cond marks conditional lower and upper bounds. These bounds reserve enough capacity for later pumps, and ζiP∈[0,1] selects the current allocation. Apply this interpolation through the penultimate pump and assign the remainder to the last pump. A fixed interval gives its single value directly. The resulting vector satisfies every individual bound and the selected total.

For a numerical illustration, individual bounds [1,5] and [2,6], total band [6,8], and coordinates (ζΣP,ζ1P)=(0.5,0.25) give pΣ=7 and (P1,P2)=(2,5). All powers in this example are in kW.

Next calculate Qi and qij from the reference plant in (2). The balances in (15)-(16) give each tank's current delivery interval [Lj,Uj]. Select total useful delivery from

[LD,UD]=[max{0,Dt-εtQ}, Dt+εtQ]∩[∑jLj, ∑jUj],dΣ=LD+(UD-LD)ζΣd.	(14f)

Here LD,UD bound aggregate delivery, dΣ is the selected total, and ζΣd∈[0,1] is its coordinate. An empty tank interval or total intersection rejects the point. Allocate dΣ among tanks using the remaining-total rule in (14e), with dj,Lj,Uj replacing pump powers and their bounds. The last tank receives the remainder. Exact aggregate demand fixes dΣ=Dt. When Rj>Sjmax, the lower bound in (16) specifies the delivery required to remove excess bookkeeping inventory. The point passes this step when that delivery fits within the tank capacities and aggregate service band.

For each delivering tank, choose one allowed building connection, assign its total dj to that connection and set its other delivery flows to zero. This gives a common representation of the equivalent allocations in (1a). A tank with no allowed building connection has zero effective delivery capacity. Calculate the successor with (4) and retain points that pass every current check, including the COP domain, edge capacities, tank capacities and shared-source limit. Checks use the mapped action and its calculated inventory directly. Future continuation is evaluated at the later acceptance stage.

Each mode with continuous coordinates uses one fixed scrambled Sobol sequence. Increase density by doubling the number of points in its prefix. Keep the mode catalogue, coordinate order, mapping and scramble fixed across density levels. Start the pool with the backup action, append legal mapped actions, and merge duplicates while retaining the first representative. The pools then satisfy

𝒜t(N)⊆𝒜t(2N),  N∈{1,2,4,8,…},K(N)=|𝒜t(N)|-1.	(14g)

Here N is the raw sample count per nonzero-dimensional mode. The pool 𝒜t(N) contains the retained current-legal actions, including At(0), and K(N) counts its other candidates. The parameter N controls sampling effort; rejection and duplicate removal determine the actual pool size. Exact duplicates in the common representation are merged. Any numerical merging tolerance is fixed across levels, and its maximum merge distance is recorded. Extending the prefix retains every existing candidate and its identifier.

An all-off mode can deliver stored heat with zero pump power. A single adjustable device receives the total remaining after fixed contributions. A zero-dimensional mode is evaluated once. Coordinates that remain constant throughout a mode can be removed when defining its dimension; a local interval collapse uses the fixed value and preserves that dimension. The resulting action distribution depends on the Sobol sequence, conditional mapping and rejection checks. Sampling records include raw attempts, unique legal candidates and rejection reasons.

Candidate density controls action coverage, and message fidelity controls score approximation. Hold 𝒜t(N) fixed to evaluate message refinement. Keep the complete evaluator H fixed and vary N to evaluate candidate coverage. The decision loss in (22) compares choices within one pool. Joint comparisons use the same measured total time, including candidate generation and current checks, and the same continuation-acceptance budget. Increasing N expands the sampling effort while retaining the specified service tolerances and reference-plant checks.

3 Physical consequences and energy messages

Suppress the current time t and candidate index within (15)-(16). For each joint action, calculate total heat power received by a tank Hj, its energy after charging and standing loss but before delivery Rj, and its lower and upper useful-delivery power bounds Lj,Uj:

Hj=∑iqij,  Rj=Sj+Δt[ηjcHj-UAj(Tjtank-Tamb)].	(15)

Lj=max{0,ηjd(Rj-Sjmax)/Δt},Uj=min{djeff,ηjd(Rj-Sjmin)/Δt}.	(16)

The effective delivery limit djeff equals dj when tank j has an allowed building connection and equals zero otherwise. A candidate passes the current physical checks when its delivery interval is nonempty and it satisfies successor-inventory, charging and shared-source limits. When Rj>Sjmax, the lower bound in (16) gives the delivery needed to restore a legal successor; the delivery capacities and other constraints determine whether this is feasible. These quantities are calculated for the joint action, so all incoming pumps share the tank's available capacity.

Let e=(u→v) denote a directed connection and τ(e) its relation type. The endpoint labels u and v identify the sender and receiver. Device indices i,j,b identify pumps, tanks and buildings, whose graph node labels are Hi, Sj and Bb. The tank node label Sj and its energy state are distinguished by context; likewise, Hi identifies a pump node, Hj in (15) is received heat power, and H names the complete evaluator. Candidate index k applies to action-dependent quantities below. The vector ℓe,k records four energy roles r=1,2,3,4: electricity, source extraction, tank-inventory contribution and useful delivery. For a charging edge,

ℓ(Hi,Sj),k=Δt [vij,kPi,k; vij,k(Qi,k-Pi,k); ηjcqij,k; 0].	(16a)

Here vij,k selects the receiving tank, Pi,k is pump electrical power, Qi,k is produced heat power, and qij,k=vij,kQi,k is charging-edge heat power. A delivery edge carries

ℓ(Sj,Bb),k=Δt [0; 0; -fjb,k/ηjd; fjb,k].	(16b)

Here fjb,k is useful delivered heat power. Multiplication by Δt gives energy, with charging and discharge efficiencies incorporated in the inventory coordinate. Each coordinate records its own resource role. Adding the third coordinates across incoming and outgoing edges reproduces the inventory balance:

Sj,k+-Sj=∑iℓ(Hi,Sj),k,3+∑bℓ(Sj,Bb),k,3  -Δt UAj(Tjtank-Tamb).	(16c)

The quantity Sj,k+ is the successor energy of tank j under candidate k, and the final subscript 3 selects an edge's inventory contribution. Unavailable edges have zero ledgers, allowing the sums to cover all device indices. Standing loss is charged once per tank. Normalize the energy coordinates by fixed positive role scales sr and add a background slot:

ze,k,r=ℓe,k,r/sr (r=1,…,4),  sr>0,  ze,k,0=1,ze,kenergy=[ze,k,1;ze,k,2;ze,k,3;ze,k,4].	(16d)

Here ℓe,k,r is energy-ledger component r, and sr has the corresponding energy units, making ze,k,r dimensionless. The vector ze,kenergy contains the four normalized energies. The background slot ze,k,0=1 retains connection context at zero flow. Its role index is zero; the physical energy roles are indexed one to four. Semicolons in bracketed expressions and the operator concat stack vectors in the stated order.

The endpoint contexts cu,k,cv,k collect states, equipment modes and locks, joint successor inventories, receiving and source margins, allowed-neighbour summaries and known future inputs. They also include the remaining horizon and relevant terminal requirements. The edge vector de contains known connection attributes. These features use information available at the decision time. Inventory fractions are normalized by each tank's upper-minus-lower bound, with absolute capacity retained as a feature. Tank delivery totals and aggregate service describe building allocation. The contexts and attributes remain fixed during refinement.

4 Coarse and complete message operators

Message refinement operates on allowed charging connections. Delivery uses the same linear representation at both fidelities. Scoring uses each tank's total dj,k=∑bfjb,k and aggregate service information. All building allocations preserving every dj,k therefore receive the same score for fixed equipment states and external inputs. This equivalence is built into both the contexts and the readout.

Write τ=τ(e). The coarse branch uses a narrow, endpoint-conditioned gate:

ge,k,rL=1+tanh(Aτ,rucu,k+Aτ,rvcv,k+Aτ,rdde+bτ,r),τ=τ(e),  r=0,…,4,  ge,k,rL∈ℝdL.	(17a)

The trainable matrices Au,Av,Ad project sender context, receiver context and edge attributes, respectively; b is the bias vector. Parameters are shared by relation τ and role r, and φ collects the coarse parameters. The gate width dL is its number of coordinates. The function tanh acts elementwise, the constant 1 is an all-ones vector, and ℝdL denotes real vectors of that width. The gate modulates the learned representation. Endpoint and static-edge projections can be cached.

ye,kL=Wτ,φLconcatr=04(ze,k,rge,k,rL),me,kL=[ze,kenergy;ye,kL],  ye,kL∈ℝdm.	(17)

Each scalar ze,k,r scales its role gate, and concatenation preserves the role order. The matrix Wτ,φL maps the resulting 5dL coordinates to the latent width dm. The coarse message me,kL joins the exact energy vector to the learned vector ye,kL. The symbol y here denotes learned message coordinates; yb,t in Part I denotes building heat receipt. The fixed role slots provide the quantity representation used by this branch.

For the complete branch, concatenate endpoint and edge conditions into pe,k and bind each role quantity to that same context:

pe,k=[cu,k;cv,k;de],ξe,k=[ze,kenergy;concatr=04(ze,k,rpe,k)].	(18)

The vector pe,k collects both endpoint contexts and the edge attributes. With ze,k,0=1, the complete-network input ξe,k retains absolute context and its products with each role quantity. These blocks represent interactions in the neural calculation. If the context vector p has width dp, the input ξ has width 4+5dp.

ye,kH=W2,τ,θReLU(W1,τ,θξe,k+b1,τ,θ)+b2,τ,θ,me,kH=[ze,kenergy;ye,kH],  ye,kH∈ℝdm.	(19)

The complete learned vector ye,kH has width dm, and me,kH joins it to the same four physical coordinates. The matrices W1,W2 and corresponding biases form the hidden and output affine layers, indexed 1 and 2. The activation is elementwise ReLU(x)=max(x,0), where x denotes a scalar activation input. These relation-specific parameters belong to θ, the frozen complete evaluator. The coarse branch combines conditioned roles linearly; the complete branch captures nonlinear interactions across roles and contexts.

The complete first layer also admits exact projection caching:

W1ξe,k+b1=Wzze,kenergy  +∑r=04ze,k,r(Urcu,k+Vrcv,k+Drde)+b1.	(19a)

The matrices Wz,Ur,Vr,Dr are blocks of W1. Relation and parameter subscripts are suppressed in (19a). The physical block acts on energy coordinates; the role-specific blocks act on sender context, receiver context and edge attributes, respectively. Both implementations cache these projections. For a role block that is identically zero for a relation, both omit its gate, projection and cache preparation. Timing then reflects the computations actually performed.

Aggregate charging messages at tanks and predict continuation cost:

hSj,kH=Uθ(cSj,k,∑i:(i,j)∈Echm(Hi,Sj),kH),JH(k)=gt(At(k))+Rθ({cHi,k}i,{hSj,kH}j,dk,Dt,wt+1:T).	(20)

hSj,kH is tank j's updated representation. The function Uθ combines its context with summed incoming messages. The readout Rθ predicts future electricity cost from pump contexts, updated tank representations, the delivery vector dk, aggregate demand Dt and the known future sequence w. The vector contains all dj,k values. The sequence wt+1:T contains the external inputs over the remaining horizon, using the terminal-boundary convention. Braces indexed by i or j collect the corresponding pump or tank inputs. The prediction target is Jcont(k)-gt(At(k)); adding exact current cost gives JH(k). Context construction, node updates and readout are shared and frozen during message replacement.

5 Message replacement and decision quality

Define eligible message positions 𝒫=Ech×{0,…,K}, with K=K(N) for the selected sampling count. Each position identifies one charging connection in one candidate. Let 𝒮⊆𝒫 be the set of positions already refined. The mixed message replaces their learned coordinates:

ye,k𝒮=ye,kL+1[(e,k)∈𝒮](ye,kH-ye,kL),me,k𝒮=[ze,kenergy;ye,k𝒮].	(21)

The indicator 1[·] equals one when its condition is true and zero otherwise. Thus ye,k𝒮 uses the complete learned vector at refined positions and the coarse vector elsewhere. The full mixed message me,k𝒮 retains the exact physical coordinates. Shared updates and readout produce J𝒮(k). An empty refinement set gives the all-coarse evaluator; refining all of 𝒫 reproduces the complete evaluator. Online execution computes complete messages as their positions are selected.

k𝒮=arg minkJ𝒮(k),  kH=arg minkJH(k),ℛH(𝒮)=JH(k𝒮)-JH(kH)≥0.	(22)

The operator arg min selects the candidate index with the smallest score, using a common rule for ties. Both minimizations run over k=0,…,K in the same pool. The indices kH and k𝒮 are the choices of the complete and mixed evaluators. The decision loss ℛH is the increase in the complete evaluator's score caused by the mixed choice. It is measured in currency and evaluates agreement with the complete model's cost preference.

minπ𝔼[ℛH(𝒮π)],  Cπ≤Cmax.	(23)

In (23), π is the computation strategy: it selects successive message groups or stops, producing 𝒮π. Part III uses the same letter for the physical control policy. The measured cumulative time Cπ covers current-step physics, contexts, coarse scoring, error prediction, gradients, scheduling, cache preparation, refinement and repeated readout. The fixed limit Cmax is the available scoring and refinement time in the same units. Each actual invocation is charged once. Total control time additionally includes candidate generation and final continuation acceptance. The expectation 𝔼 averages over the declared state/candidate distribution and strategy randomness. Complete reference scores provide offline supervision; online refinement uses the scores and features available at its current stage.

6 Training for decision quality

Train the complete evaluator H on the verified continuation bills in (14a), then freeze its parameters θ and inference state. For each candidate pool 𝒜t(N), its preferred index kH is defined by (22). The trainable coarse parameters φ are fitted to favour candidates with low teacher-assessed cost. Soft candidate weights provide a differentiable training objective:

p𝒮(k)=exp[-J𝒮(k)/(sJτsel)]∑k′=0K(N)exp[-J𝒮(k′)/(sJτsel)],ℒcoarse(φ)=𝔼xt,N,𝒮[∑k=0K(N)p𝒮(k)sg(JH(k)-JH(kH))sJ].	(24)

Here p𝒮(k) is the scalar training weight assigned to candidate k from its mixed score. The weights sum to one, and k' is the dummy candidate index in the denominator. The operator exp is the natural exponential. The fixed scale sJ>0 has currency units and is chosen from training data for use across states and candidate densities. The dimensionless temperature τsel>0 controls the concentration of weights on low scores; sel identifies selection temperature, while τ denotes an edge relation type.

The dimensionless loss ℒcoarse averages over the training distribution of states xt, raw sampling counts N and refinement sets 𝒮. As in (14g), N counts raw points per nonzero-dimensional mode, and K(N)+1 is the actual pool size. Each configuration contributes the weighted sum shown in (24). The scale and temperature are fixed before final evaluation.

The stop-gradient operator sg returns the teacher cost gaps unchanged in the forward calculation and holds them constant during differentiation. Complete-message parameters, node updates and readout parameters are frozen. Gradients pass through the shared update and readout functions to the unrefined coarse messages. The physical action, exact current cost, teacher choice kH and refinement set 𝒮 are fixed inputs for each update. Teacher scores are calculated offline for supervision.

Multiplying the single-configuration training loss by its currency scale gives its cost interpretation:

sJℓcoarse=∑k=0K(N)p𝒮(k)JH(k)-JH(kH).	(24a)

Here ℓcoarse is the dimensionless single-configuration loss inside the expectation in (24). The quantity sJℓcoarse is the expected teacher score under the soft weights, measured above the teacher's best score in that pool. Each JH(k) includes exact current electricity cost gt(At(k)) and predicted future cost from (20). Training therefore accounts for both immediate electricity use and later recovery. The operating objective remains the complete electricity bill in (11), subject to the service and terminal requirements.

For example, teacher scores (100,101,120) for k=0,1,2 give cost gaps (0,1,20). The scaled loss is sJℓcoarse=p𝒮(1)+20p𝒮(2): the same weight on the third candidate contributes twenty times as much loss as on the second. The weights and loss are invariant to a common offset added to every mixed score. Training focuses on the cost consequence of the candidate choice.

Equation (24) is the main coarse-branch training objective. An optional short preliminary fit of yL to frozen yH initializes the branch, followed by decision training. Persistent auxiliary losses define separately evaluated variants. Training cases include the all-coarse configuration and partial message replacements representative of deployment budgets. Sets collected from an adaptive scheduler use its online-visible information and remain fixed within each training update.

Deployment selects the hard choice k𝒮 in (22). Temperature, checkpoint and optional initialization are chosen using held-out hard decision loss ℛH, with results reported by candidate density and measured computation budget. Finite temperature assigns soft weight to higher-cost candidates, so positive soft loss can coexist with the correct hard choice. Score-gap scaling changes the effective temperature, and concentrated incorrect weights can produce small gradients. Validation therefore tracks score-gap scale and hard decisions alongside training loss. Full refinement gives zero gradient to φ and supplies a consistency check.

Equation (24a) measures a teacher-assessed cost difference during training. End-to-end evaluation records accepted-plan bills and backup use alongside teacher-relative decision loss. Together, these measurements show how teacher prediction accuracy, candidate coverage and continuation acceptance affect the executed schedule and its electricity bill.

After fixing the coarse branch, train an error predictor on paired coarse and complete messages:

be,k≈‖ye,kH-ye,kL‖2,  be,k≥0.	(25)

The nonnegative estimate be,k predicts the latent residual norm using coarse-visible information. The notation ‖·‖2 is the Euclidean norm, and the hat marks an estimate. Refit this predictor after coarse-branch training, then reassess the priority in (28) under the same measured budget. This calibration follows the residual magnitudes and score sensitivities of the trained branch. All evaluators and prediction models remain fixed during each control call.

7 Sensitivity and refinement priority

For two distinct candidate indices a,b, define their mixed-score difference and the learned-vector replacement at an unrefined position:

Fab=J𝒮(a)-J𝒮(b),  δe,k=ye,kH-ye,kL,ge,kab=∂Fab∂ye,k𝒮,  ΔFab≈(ge,kab)⊤δe,k,|(ge,kab)⊤δe,k|≤‖ge,kab‖2‖δe,k‖2.	(26)

Here Fab is the score difference, δe,k is the complete-minus-coarse learned vector, and ΔFab is the change caused by replacing this position while holding other messages fixed. The vector ge,kab is the gradient of the difference with respect to the current mixed message. The notation ∂/∂y denotes differentiation with respect to vector y, and ⊤ denotes transpose. Candidate indices a,b in this subsection identify the compared actions. The first-order contribution is their gradient-residual inner product. Cauchy-Schwarz bounds its magnitude by the product of the two norms. Residual direction, nonlinear response and interactions among replacements determine the full score change.

Each candidate score depends on its own input graph, with neural parameters shared across candidates. Its message gradient therefore has the following form:

ge,kab=∂J𝒮(k)/∂ye,k𝒮  if k=a,ge,kab=-∂J𝒮(k)/∂ye,k𝒮  if k=b,ge,kab=0  if k∉{a,b}.	(27)

Every comparison containing candidate k uses the same message-gradient norm. Comparisons excluding k give a zero vector of the learned-message width. One sensitivity norm per candidate-message position is therefore sufficient for the priority calculation:

ηe,k=1[k∈𝒦ℓ]‖∂J𝒮(k)∂ye,k𝒮‖2be,k,  (e,k)∈𝒫∖𝒮.	(28)

The set 𝒦ℓ contains candidates considered in refinement round ℓ, including the current leader, nearby competitors and candidates selected by a fixed coverage rule. The separate index r continues to identify message roles. The priority ηe,k multiplies local score sensitivity by predicted latent error, estimating the scale of a message's effect on the score. This priority is distinct from the physical charging and delivery efficiencies. Gradients are evaluated at neural message inputs while the candidate's equipment decisions remain fixed. The set 𝒫∖𝒮 contains positions still using coarse messages.

8 Message groups and incremental execution time

A proposed group Ω is a subset of eligible candidate-message positions. The group catalogue can pair the old and new receivers of a reassigned pump and also covers other allowed connections, including zero-flow backgrounds. Refinement evaluates the specified candidate actions in greater detail. For each group, identify the new positions and their shared update dependencies:

ΔΩ=Ω∖𝒮,𝒱Ω={(v,k):∃(e,k)∈ΔΩ, dst(e)=v},𝒦Ω={k:∃e, (e,k)∈ΔΩ}.	(28a)

The set ΔΩ contains the group's positions awaiting refinement, after removing those in 𝒮. The set 𝒱Ω contains the affected receiver-candidate pairs, and 𝒦Ω contains the affected candidates. The function dst(e) returns the receiving node of edge e, and ∃ means that at least one matching position exists. Empty increments are skipped. Cached incoming-message sums are updated by adding the replacement differences:

Mv,knew=Mv,kold+∑(e,k)∈ΔΩ:dst(e)=v(me,kH-me,kL).	(28b)

Mv,k is the current sum of incoming message vectors at receiver v for candidate k. Superscripts old and new identify the values before and after the group update. Messages sharing a receiver-candidate pair require one aggregate update and one node update. Each candidate retains its own cached aggregate and representation. The affected readouts are then recalculated using the existing physical balances and fixed contexts.

Before computing the complete messages, describe the proposed incremental workload by the count vector χΩ:

χΩ=[{nτH}τ;{nαcache}α;{nνU}ν;nR;nread;dmaxΔ].	(28c)

Here nτH counts new complete messages for each relation type. The quantity nαcache counts missing projections for each cache category α, and nνU counts deduplicated receiver-candidate updates for each node type ν. The count nR records readouts, and nread records the node representations accessed by those readouts, including full-graph accesses when required. The quantity dmaxΔ is the largest number of new messages entering any one receiver-candidate pair. All workload features are available before refinement.

crefine(Ω|𝒮)=Treeω(χΩ),  ΔΩ≠∅.	(28d)

Here Treeω is a fitted regression tree. Its parameters ω specify split features, thresholds and leaf predictions. The output crefine estimates incremental execution time for the group, in the same units as the computation budget. The symbol ∅ denotes the empty set. Conditioning on 𝒮 is shorthand for the current refinement and cache state; the implementation tracks the cache explicitly.

Calibration fixes the hardware, precision, network widths and execution schedule. Offline measurements cover complete incremental batches, including cache preparation, messages, aggregation, node updates and readouts. Synchronized timings are collected after warm-up. The tree uses absolute-error splits and median batch times at its leaves. Validation selects tree depth and minimum leaf samples, with additional checks on unseen workload configurations. Workloads outside the measured profile trigger further calibration. Feature construction and tree-query time are included in scheduling overhead.

The model follows the measurement-based latency approach of nn-Meter and profiles this evaluator's incremental batches directly. Measuring whole batches captures the effects of shared work, batching and parallel execution.

I(Ω|𝒮)=∑(e,k)∈ΔΩηe,kcrefine(Ω|𝒮).	(29)

The group score I divides summed new-message priorities by predicted execution time. It ranks nonempty groups with positive predicted time by priority per unit of computation. The scheduler chooses a group that fits the predicted remaining budget, executes it, records actual elapsed time and updates the affected scores. It repeats for a bounded number of rounds, reserving time for final output. Measured accounting includes error prediction, gradients, tree queries, selection and output. The median latency estimates guide allocation; actual budget use and overruns are reported. When the available time falls short of basic scoring, the controller uses the still-valid backup and records a budget failure.

9 Score accuracy and control execution

A uniform score-error bound relates mixed-score accuracy to teacher-relative decision loss:

maxk|J𝒮(k)-JH(k)|≤ε  ⇒  ℛH(𝒮)≤2ε.	(30)

Here ε is a nonnegative currency-valued bound on absolute score error over every candidate in the same pool. It is separate from the physical service tolerances in (10). Insert the mixed scores between the selected and teacher-best scores: the two approximation terms are each at most ε, and the mixed-choice term is nonpositive. This gives (30). A teacher best-to-second-best gap exceeding 2ε preserves the chosen index. Applying this result requires the stated uniform score-error bound. Complete scores provide an offline check of that condition.

A verified initial backup trajectory is an input to the control procedure. After scoring, the acceptance procedure tries a bounded number of candidates in score order, constructs their remaining trajectories and checks all equipment, service and terminal requirements. An accepted trajectory supplies the next control action and updates the backup. If the search yields no accepted trajectory, the controller executes the first action of the existing backup and retains its suffix. With the deterministic reference plant and future inputs unchanged, this suffix remains a known feasible continuation.

The index k𝒮 identifies the score-preferred candidate; the acceptance result identifies the executed action. Evaluation reports teacher-relative decision quality, trajectory acceptance, backup use, executed electricity bills and total computation time. Unresolved continuation searches are recorded as UNKNOWN. These results connect message approximation to the behaviour of the complete controller.